Midi Implementation

Function	Transmitted		Recognized		Remarks

Note ON	9nH, v = 1 -127		9nH, v = 1 - 127

Note OFF	9nH, v = 0		9nH, v = 0 or 8nH

Contr.Change

Mod.Wheel	no			1			7 Bit

Glide		5 + 37			5 + 37			10 Bit

Mod.Vco	6 + 38			6 + 38			10 Bit (64+00=OFF)

Pulse		8 x 40			8 + 40			10 Bit (64+00=OFF)

Rate1		9 + 41			9 + 41			10 Bit

Interval 1	10 + 42			10 + 42			14 Bit

Interval 2	11 + 43			11 + 43			14 Bit

Rate2		12 + 44			12 + 44			10 Bit

Level VCO1	13 + 45			13 + 45			10 Bit

Level VCO2	14 + 46			14 + 46			10 Bit

Level VCO3	15 + 47			15 + 47			10 Bit

Level Noise	16 + 48			16 + 48			10 Bit

Audio In	17 + 49			17 + 49			10 Bit

Mod.VCF	18 + 50			18 + 50			10 Bit (64+00=OFF)

Mod.VCA	19 + 51			19 + 51			10 Bit (64+00=OFF)

KeyFollow	20 + 52			20 + 52			10 Bit

Cutoff		21+ 53			21 + 53			14 Bit

Emphasis	22 + 54			22 + 54			10 Bit

Contour		23 + 55			23 + 55			10 Bit

Attack1		24 + 56			24 + 56			10 Bit

Decay1		25 + 57			25 + 57			10 Bit

Sustain1	26 + 58			26 + 58			10 Bit

Release1	27 + 59			27 + 59			10 Bit

Attack2		28 + 60			28 + 60			10 Bit

Decay2		29 + 61			29 + 61			10 Bit

Sustain2	30 + 62			30 + 62			10 Bit

Release2	31 + 63			31 + 63			10 Bit

Octave1		80			80			2 Bit

Wave1		81			81			2 Bit

Octave2		82			82			2 Bit

Wave2		83			83			2 Bit

Octave3		84			84			2 Bit

Wave3		85			85			2 Bit

Wave.Mod1	86			86			3 Bit

Wave.Mod2	87			87			3 Bit

PRG Change

User Memory	01 - 49			01 - 49

Presets		50 - 99			50 - 99

Setting system parameters

To alter the MFB-SYNTH II´s basic settings press Tempo followed by Record. The display has to show two dots. Use C1/1...C3/25 for the following edits. The bold printed settings are factory preset.

C1/1	MOD.VCO	0 = LFO1	1 = LFO2

C#1/2	PWM	0 = LFO1	1 = LFO2

D1/3	MOD.VCF	0 = LFO1	1 = LFO2

D#1/4	MOD.VCF	0 = VCO1	1 = VCO2	2 = VCO3

E1/5	SYNC	0 = internal	1 = external

F1/6	Dynamic VCF 	0 = Off	1 = On

F#1/7	Dynamic VCA	0 = Off	1 = On

G1/8	Pitch Bender	0 = +/- 1 semi	1 = +/- 3 semi	2 = +/- 1 octave

G#1/9	Mod.Wheel	0 = VCO	1 = VCF	2 = VCA

A1/10	CV In	0=Off	1=VCO	2=VCF

A#1/11	PRG Change	0=Off	1=MIDI Out

B1/12	PRG Change	0=Off	1=MIDI In

C2/13	Contr. Change	0=Off	1=MIDI Out

C#2/14 	Contr. Change	0=Off	1=MIDI In

D2/15	Sequence Out	0=Off	1=MIDI Out

B2/24	CV In Tune	minus 1

C3/25	CV In Tune	plus 1

Parameter visualisation, MIDI-Controllers

Each stored parameter value in a programm can be viewed. Press and hold Shift and Tempo followed by one of the 25 note buttons.

C1/01		RATE LFO1			C#1/02		PULSE

D1/03		MOD.VCO			D#1/04		GLIDE

E1/05		RATE LFO2			F1/06		INTERVAL VCO3

F#1/07		INTERVAL VCO2		G1/08		MIXER NOISE

G#1/09		MIXER VCO3			A1/10		MIXER VCO2

A#1/11		MIXER VCO1			B1/12		MOD.VCO

C2/13		MOD.VCF			C#2/14		ATTACK ADSR2

D2/15		ATTACK ADSR1		D#2/16		KEY

E2/17		DECAY ADSR2			F2/18		DECAY ADSR1

F#2/19		CUTOFF			G2/20		SUSTA1N ADSR2

G#2/21		SUSTAIN ADSR2		A2/22		EMPHASIS

A#2/23		RELEASE ADSR2		A2/24		RELEASE ADSR1

C3/25		CONTOUR

Presets (Preset + Keyboard)

No.	Name	Category

01	Typical	Lead

02	Metasync	Lead

03	Bobcat	Lead

04	Attack Bass	Bass

05	Mind Scanner	FX

06	Small PWM	Lead / Bass

07	FM Bass	Bass / Seq

08	Random Step-VCF	Bass/ Seq

09	Channel Cord	Seq

10	Sweep Ring	Seq / Lead

11	Glider	Lead / Seq

12	Dual Ring	Seq / Lead

13	Slide Saw	Lead

14	LFO Liner	Bass / Seq

15	90s Detuned Hook	Seq / Lead

16	Thin & Ugly   	Seq

17	Less Paul	"Guitar"

18	Messing Flute	Lead

19	DX Plus	Lead / Seq

20	Soft Drivin	Lead

21	Phatt Bass	Bass / Sub

22	Full Square	Lead / Seq

23	S&H Filter	Lead

24	Dirty Larry	Lead / Seq

25	Sharp Overtone	Lead

Presets (Shift + Preset + Keyboard)

No.	Name	Category

01	Soft Jazzy	Organ

02	Mono Divider	Organ

03	Unmelodic Blaster   	FX / Bass

04	Cosmic Backround	FX

05	Stepping Filter	Seq

06	High Tizzle	Seq

07	Soft High Sync	Lead

08	Falling Formant	FX

09	Melodymaker	Lead

10	Simply Nice	Lead

11	Disintegrated Voice	FX

12  	Metallic Voice	FX / Seq

13	Tok Bass	Bass

14	Little Sweeper	Seq

15	Soft DX	Seq

16	Tackle Sync	Seq / Lead

17	Full Bandwide	Lead / Bass

18	Growl Bass	Bass

19	Drush Tone	Seq

20	Analog SID-Chip	Lead / FX

21	Cowbell (ca. D1)	Drum

22	Drive Kick (C1-C2) 	Drum

23	closed Hihat	Drum

24	Dance Kick	Drum

25	Snare (C1-C2)      	Drum