MIDI Implementation

Function	Transmitted		Recognized		Remarks

Note ON	9nH, v = 1 -127	9nH, v = 1 - 127

Note OFF	9nH, v = 0		9nH, v = 0 or 8nH

Contr.Change

Mod.Wheel	no			1			7 Bit

Interval 1	8 + 40			8 + 40			14 Bit

Interval 2	9 + 41			9 + 41			10 Bit

Rate		10 x 42		10 + 42		10 Bit

Level VCO1	11			11			7 Bit

Level VCO2	12			12			7 Bit

Level VCO3	13			13			7 Bit

Depth		14 + 46		14 + 46		12 Bit

Cutoff		15 + 47		15 + 47		14 Bit

Emphasis	16			16			7 Bit

Contour	17 + 49		17 + 49		12 Bit

Attack1	18 + 50		18 + 50		10 Bit

Decay1	19 + 51		19 + 51		10 Bit

Sustain1	20 + 52		20 + 52		10 Bit

Attack2	21 + 53		21 + 53		10 Bit

Decay2	22 + 54		22 + 54		10 Bit

Sustain2	23 + 55		23 + 55		10 Bit

Tune		24 + 56		no

Octave1	80			80			2 Bit

Wave1		81			81			2 Bit

Octave2	82			82			2 Bit

Wave2		83			83			2 Bit

Glide		84			84			2 Bit

Wave3		85			85			2 Bit

Program Change

User Memory	00 - 83			00 - 83

Sequencer	84 – 103		84 - 103

System settings

There a few basic system settings for the SYNTH LITE II. Press and hold RECORD in combination with th ebelow listed buttons to edit:

# Velocity

Octave (OSC1)     0 = Off	1 = VCF	2 = VCA	3 = VCF/VCA

# MIDI Clock

Wave (OSC1)       0 = internal	1 = external

# Keyboard Follow

Octave(OSC2)      0 = Off	1 = ¼ 		2 = ½ 		3 = ON

# Release = Decay

Wave(OSC2)	    0 = OFF	1 = ADSR1	2 = ADSR2	3 = ADSR1+2

# MIDI In/Out

Glide/Shift	    0 = Thru	1 =ProgChg  2 = ConChg	3 = both

# Mod. Wheel

Wave(Mod)	    0 = Off	1 = VCO	2 = VCF	3 = VCO/VCF

Polyphony - Cascading multiple LITE II modules.

Although each SYNTH LITE II is monophic, up to five modules can be cascaded to act as a polyphonic synthesizer. Connect the keyboard /sequencer to the first module and chain the following modules in series. This is how it works: The last LITE II will play all incoming MIDI notes, the second last has to play every second note and send out the other one, the third plays every third note and sends the other two notes and so on. Switch on your Lite II modules while holding:

- OCTAVE (OSC1) 	this LITE II will play all notes

(This is the standard setting)

- WAVEFORM (OSC 1)	this LITE II will play every second note and

send the other to its MIDI output

- OCTAVE (OSC2 )	this LITE II will play every third note and send

the others to its MIDI output.

- WAVEFORM (OSC2)	this LITE II will play every fouth note and send

the others to its MIDI output.

- GLIDE	this LITE II will play every fifth note and send

the others to its MIDI output.

Tip: You can try to use other keyboards in the last position to split polyphonic sounds across different modules.

### Presets

The listed presets were programmed using the the following system parameters:

Velocity 		= 	2 	VCA

Key Follow  		= 	3 	On

Release/Decay 	= 	3	1+2 On

Mod-Wheel 		=	2 	VCF

Changing any of the above system settings can affect the way the presets sound or are intended to sound. This is especially true for the Key Follow parameter setting.

### Classic Synthsounds

- PWM (reserve memory)

- Improved Saw

- FullPulseRing

- Detuned

- Dualik

- Fat Saw

### Lead/Seq – Noisy & Ringing

11	Gribblin

12	ResoRing

13	Mezzotine

14	Noislide

15	Sharp

16	Sharper

### Lead/Seq – Sync

- Slow Stepper

- Pulsing

- Medium

- Softy

- Saturation

- Crunk (>C4)

### Lead/Seq – Normal

31	Coastslider

32	Soft detune

33	Thrilling

34 	Standard Lux

35	Cheapy

36 	Dustslider

### Bass – Bigger

- Loher

- Bride

- Skware

- Grummling

- Grolling

- Softeq

### Bass – Reso

51	Silver

52	San ReMo

53	Rawring

54 	Rezzo

55	LFO-Liner

56	Dancer

### Percussion

- Little Drum

- Digishot

- Resect 1

- Resect 2

- Co-Bell C4

- Zapp